TASK 1:

Testbench:

`timescale 1ns/1ps

module testbench;

reg pclk;

reg presetn;

reg transfer;

reg read;

reg write;

reg [8:0] apb_write_paddr;

reg [7:0] apb_write_data;

reg [8:0] apb_read_paddr;

wire pslverr;

reg [3:0] test_case;

wire [7:0] apb_read_data_out;

top dut (

.pclk(pclk),

.presetn(presetn),

.transfer(transfer),

.read(read),

.write(write),

.apb_write_paddr(apb_write_paddr),

.apb_write_data(apb_write_data),

.apb_read_paddr(apb_read_paddr),

.pslverr(pslverr),

.apb_read_data_out(apb_read_data_out)

);

initial begin

pclk = 1'b0;

forever #5 pclk = ~pclk;

end

integer pass_count;

integer fail_count;

task reset_dut;

begin

presetn = 1'b0;

transfer = 1'b0;

read = 1'b0;

write = 1'b0;

apb_write_paddr = 9'h000;

apb_write_data  = 8'h00;

apb_read_paddr  = 9'h000;

#20;

presetn = 1'b1;

@(posedge pclk);

#1;

end

endtask

task apb_write_transaction;

input [8:0] address;

input [7:0] data;

begin

@(negedge pclk);

transfer = 1'b1;

write = 1'b1;

read = 1'b0;

apb_write_paddr = address;

apb_write_data  = data;

// IDLE -> SETUP

@(posedge pclk);

#1;

// SETUP -> ENABLE

@(posedge pclk);

#1;

// Wait until transfer completes

while ((dut.master_inst.state == 2'b10) &&

(dut.master_inst.pready == 1'b0)) begin

@(posedge pclk);

#1;

end

@(negedge pclk);

transfer = 1'b0;

write = 1'b0;

end

endtask

task apb_read_transaction;

input [8:0] address;

output [7:0] data;

begin

@(negedge pclk);

transfer = 1'b1;

write = 1'b0;

read = 1'b1;

apb_read_paddr = address;

// IDLE -> SETUP

@(posedge pclk);

#1;

// SETUP -> ENABLE

@(posedge pclk);

#1;

// Wait until transfer completes

while ((dut.master_inst.state == 2'b10) &&

(dut.master_inst.pready == 1'b0)) begin

@(posedge pclk);

#1;

end

data = apb_read_data_out;

@(negedge pclk);

transfer = 1'b0;

read = 1'b0;

end

endtask

task TC1_basic_write;

begin

$display("");

$display("TC1 - BASIC WRITE OPERATION");

reset_dut;

apb_write_transaction(9'h005, 8'hAA);

#2;

if (dut.slave1_inst.memory[8'h05] === 8'hAA) begin

$display("TC1 PASS");

$display("Address  = 9'h005");

$display("Data     = 8'hAA");

$display("Slave 1  = SELECTED");

$display("Memory[5] = %h", dut.slave1_inst.memory[8'h05]);

pass_count = pass_count + 1;

end

else begin

$display("TC1 FAIL");

$display("Expected memory[5] = AA");

$display("Actual   memory[5] = %h",

dut.slave1_inst.memory[8'h05]);

fail_count = fail_count + 1;

end

end

endtask

task TC2_basic_read;

reg [7:0] read_data;

begin

$display("");

$display("TC2 - BASIC READ OPERATION");

reset_dut;

// First write AA to address 005

apb_write_transaction(9'h005, 8'hAA);

// Now read address 005

apb_read_transaction(9'h005, read_data);

#2;

if (read_data === 8'hAA) begin

$display("TC2 PASS");

$display("Address     = 9'h005");

$display("Expected    = 8'hAA");

$display("Read Data   = 8'h%h", read_data);

pass_count = pass_count + 1;

end

else begin

$display("TC2 FAIL");

$display("Expected = 8'hAA");

$display("Actual   = 8'h%h", read_data);

fail_count = fail_count + 1;

end

end

endtask

task TC3_address_decoding;

begin

$display("");

$display("TC3 - ADDRESS DECODING / SLAVE SELECTION");

reset_dut;

@(negedge pclk);

transfer = 1'b1;

write = 1'b1;

read = 1'b0;

apb_write_paddr = 9'h005;

apb_write_data  = 8'hA5;

@(posedge pclk);

#1;

if ((dut.master_inst.psel1 === 1'b1) &&

(dut.master_inst.psel2 === 1'b0)) begin

$display("Slave 1 selection PASS");

$display("Address 9'h005 -> PSEL1 = 1, PSEL2 = 0");

end

else begin

$display("Slave 1 selection FAIL");

fail_count = fail_count + 1;

end

// Finish transfer

@(posedge pclk);

#1;

@(negedge pclk);

transfer = 1'b0;

write = 1'b0;

@(negedge pclk);

transfer = 1'b1;

write = 1'b1;

read = 1'b0;

apb_write_paddr = 9'h085;

apb_write_data  = 8'h5A;

@(posedge pclk);

#1;

if ((dut.master_inst.psel1 === 1'b0) &&

(dut.master_inst.psel2 === 1'b1)) begin

$display("Slave 2 selection PASS");

$display("Address 9'h085 -> PSEL1 = 0, PSEL2 = 1");

end

else begin

$display("Slave 2 selection FAIL");

fail_count = fail_count + 1;

end

@(posedge pclk);

#1;

@(negedge pclk);

transfer = 1'b0;

write = 1'b0;

#2;

if ((dut.slave1_inst.memory[8'h05] === 8'hA5) &&

(dut.slave2_inst.memory[8'h85] === 8'h5A)) begin

$display("TC3 PASS");

$display("Slave 1 memory[05] = %h",

dut.slave1_inst.memory[8'h05]);

$display("Slave 2 memory[85] = %h",

dut.slave2_inst.memory[8'h85]);

pass_count = pass_count + 1;

end

else begin

$display("TC3 FAIL");

fail_count = fail_count + 1;

end

end

endtask

task TC4_write_wait_state;

begin

$display("");

$display("TC4 - WRITE WITH WAIT STATES");

reset_dut;

@(negedge pclk);

transfer = 1'b1;

write = 1'b1;

read = 1'b0;

apb_write_paddr = 9'h010;

apb_write_data  = 8'hBB;

// IDLE -> SETUP

@(posedge pclk);

#1;

// SETUP -> ENABLE

@(posedge pclk);

#1;

// Force ready LOW to simulate slave wait states

force dut.pready = 1'b0;

$display("Simulating wait state...");

repeat (3) begin

@(posedge pclk);

#1;

if (dut.master_inst.state !== 2'b10) begin

$display("TC4 FAIL: Master left ENABLE during wait");

fail_count = fail_count + 1;

release dut.pready;

disable TC4_write_wait_state;

end

end

$display("Master remained in ENABLE during wait.");

// Release ready

release dut.pready;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

@(negedge pclk);

transfer = 1'b0;

write = 1'b0;

#2;

if (dut.slave1_inst.memory[8'h10] === 8'hBB) begin

$display("TC4 PASS");

$display("Address = 9'h010");

$display("Data    = 8'hBB");

$display("Wait states successfully handled.");

pass_count = pass_count + 1;

end

else begin

$display("TC4 FAIL");

$display("Expected memory[10] = BB");

$display("Actual memory[10] = %h",

dut.slave1_inst.memory[8'h10]);

fail_count = fail_count + 1;

end

end

endtask

task TC5_read_wait_state;

reg [7:0] read_data;

begin

$display("");

$display("TC5 - READ WITH WAIT STATES");

reset_dut;

// Put known data into address 010

apb_write_transaction(9'h010, 8'hBB);

@(negedge pclk);

transfer = 1'b1;

write = 1'b0;

read = 1'b1;

apb_read_paddr = 9'h010;

// IDLE -> SETUP

@(posedge pclk);

#1;

// SETUP -> ENABLE

@(posedge pclk);

#1;

// Simulate wait states

force dut.pready = 1'b0;

$display("Simulating read wait state...");

repeat (3) begin

@(posedge pclk);

#1;

if (dut.master_inst.state !== 2'b10) begin

$display("TC5 FAIL: Master left ENABLE during wait");

fail_count = fail_count + 1;

release dut.pready;

disable TC5_read_wait_state;

end

end

$display("Master remained in ENABLE during wait.");

release dut.pready;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

read_data = apb_read_data_out;

@(negedge pclk);

transfer = 1'b0;

read = 1'b0;

#2;

if (read_data === 8'hBB) begin

$display("TC5 PASS");

$display("Address   = 9'h010");

$display("Read Data = 8'hBB");

pass_count = pass_count + 1;

end

else begin

$display("TC5 FAIL");

$display("Expected = 8'hBB");

$display("Actual   = 8'h%h", read_data);

fail_count = fail_count + 1;

end

end

endtask

task TC6_error_handling;

begin

$display("");

$display("TC6 - ERROR HANDLING / PSLVERR");

reset_dut;

apb_write_transaction(9'h1FF, 8'hFF);

#2;

if (pslverr === 1'b1) begin

$display("TC6 PASS");

$display("PSLVERR correctly asserted.");

$display("Invalid address = 9'h1FF");

pass_count = pass_count + 1;

end

else begin

$display("TC6 FAIL");

$display("Expected PSLVERR = 1");

$display("Actual PSLVERR   = %b", pslverr);

$display("");

$display("REASON:");

$display("9'h1FF is decoded to Slave 2.");

$display("Slave 2 receives paddr[7:0] = 8'hFF.");

$display("The slave considers 8'hFF valid.");

fail_count = fail_count + 1;

end

end

endtask

task TC7_burst_transfers;

begin

$display("");

$display("TC7 - BURST / BACK-TO-BACK TRANSFERS");

reset_dut;

// Keep transfer asserted between transactions

transfer = 1'b1;

@(negedge pclk);

write = 1'b1;

read = 1'b0;

apb_write_paddr = 9'h001;

apb_write_data  = 8'h11;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

@(negedge pclk);

apb_write_paddr = 9'h002;

apb_write_data  = 8'h22;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

@(negedge pclk);

apb_write_paddr = 9'h003;

apb_write_data  = 8'h33;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

@(negedge pclk);

write = 1'b0;

read = 1'b1;

apb_read_paddr = 9'h001;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

if (apb_read_data_out !== 8'h11)

$display("TC7 READ 001 FAIL");

else

$display("TC7 READ 001 PASS");

@(negedge pclk);

apb_read_paddr = 9'h002;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

if (apb_read_data_out !== 8'h22)

$display("TC7 READ 002 FAIL");

else

$display("TC7 READ 002 PASS");

@(negedge pclk);

apb_read_paddr = 9'h003;

@(posedge pclk);

#1;

@(posedge pclk);

#1;

if (apb_read_data_out !== 8'h33)

$display("TC7 READ 003 FAIL");

else

$display("TC7 READ 003 PASS");

@(negedge pclk);

transfer = 1'b0;

read = 1'b0;

write = 1'b0;

#2;

if ((dut.slave1_inst.memory[8'h01] === 8'h11) &&

(dut.slave1_inst.memory[8'h02] === 8'h22) &&

(dut.slave1_inst.memory[8'h03] === 8'h33)) begin

$display("TC7 PASS");

$display("Back-to-back transfers completed correctly.");

pass_count = pass_count + 1;

end

else begin

$display("TC7 FAIL");

fail_count = fail_count + 1;

end

end

endtask

task TC8_out_of_range;

begin

$display("");

$display("TC8 - OUT OF RANGE ADDRESS");

reset_dut;

apb_write_transaction(9'h1FF, 8'hFF);

#2;

if (pslverr === 1'b1) begin

$display("TC8 PASS");

$display("PSLVERR asserted for invalid address.");

pass_count = pass_count + 1;

end

else begin

$display("TC8 FAIL");

$display("Expected PSLVERR = 1");

$display("Actual PSLVERR   = %b", pslverr);

$display("");

$display("Current RTL maps 9'h1FF to Slave 2");

$display("with local address 8'hFF.");

fail_count = fail_count + 1;

end

end

endtask

task TC9_reset_behavior;

begin

$display("");

$display("TC9 - RESET BEHAVIOR");

// Assert reset

presetn = 1'b0;

transfer = 1'b0;

read = 1'b0;

write = 1'b0;

#20;

#1;

if ((dut.master_inst.state === 2'b00) &&

(dut.master_inst.psel1 === 1'b0) &&

(dut.master_inst.psel2 === 1'b0) &&

(dut.master_inst.penable === 1'b0)) begin

$display("TC9 PASS");

$display("Master returned to IDLE.");

$display("PSEL1 = 0");

$display("PSEL2 = 0");

$display("PENABLE = 0");

pass_count = pass_count + 1;

end

else begin

$display("TC9 FAIL");

$display("Reset did not return signals to default.");

fail_count = fail_count + 1;

end

// Release reset

presetn = 1'b1;

@(posedge pclk);

#1;

end

endtask

task TC10_randomized;

integer i;

reg random_write;

reg [8:0] random_address;

reg [7:0] random_data;

reg [7:0] random_read_data;

begin

$display("");

$display("TC10 - RANDOMIZED TRANSACTIONS");

reset_dut;

for (i = 0; i < 20; i = i + 1) begin

random_write   = $random;

random_address = $urandom_range(0, 8'hFF);

random_data    = $urandom_range(0, 8'hFF);

if (random_write) begin

apb_write_transaction(

random_address,

random_data

);

$display(

"Random TX %0d: WRITE Addr=%h Data=%h",

i + 1,

random_address,

random_data

);

end

else begin

apb_read_transaction(

random_address,

random_read_data

);

$display(

"Random TX %0d: READ Addr=%h Data=%h",

i + 1,

random_address,

random_read_data

);

end

end

$display("20 randomized transactions completed.");

// If simulation survived all 20 transactions,

// consider the randomized stress test successful.

$display("TC10 PASS");

pass_count = pass_count + 1;

end

endtask

initial begin

// Initialize counters

pass_count = 0;

fail_count = 0;

// Initialize test-case marker

test_case = 0;

// Initial values

pclk = 1'b0;

presetn = 1'b0;

transfer = 1'b0;

read = 1'b0;

write = 1'b0;

apb_write_paddr = 9'h000;

apb_write_data  = 8'h00;

apb_read_paddr  = 9'h000;

// Allow initialization

#10;

// TC1

test_case = 1;

TC1_basic_write;

// TC2

test_case = 2;

TC2_basic_read;

// TC3

test_case = 3;

TC3_address_decoding;

// TC4

test_case = 4;

TC4_write_wait_state;

// TC5

test_case = 5;

TC5_read_wait_state;

// TC6

test_case = 6;

TC6_error_handling;

// TC7

test_case = 7;

TC7_burst_transfers;

// TC8

test_case = 8;

TC8_out_of_range;

// TC9

test_case = 9;

TC9_reset_behavior;

// TC10

test_case = 10;

TC10_randomized;

// FINAL RESULTS

#20;

$display("");

$display("====================================");

$display("        APB TEST RESULTS");

$display("====================================");

$display("TOTAL PASSED = %0d", pass_count);

$display("TOTAL FAILED = %0d", fail_count);

if (fail_count == 0)

$display("ALL TEST CASES PASSED");

else

$display("SOME TEST CASES FAILED - CHECK RTL");

$finish;

end

Endmodule

TC1 – Basic Write Operation

Tests whether the master can successfully write 8'hAA to address 9'h005 and the data is stored correctly in Slave 1 memory.

TC2 – Basic Read Operation

Tests whether the master can read previously stored data (8'hAA) from address 9'h005 and receive the correct value.

TC3 – Address Decoding / Slave Selection

Tests whether the master correctly selects Slave 1 for lower addresses and Slave 2 for higher addresses using the address MSB.

TC4 – Write with Wait States

Tests whether the master remains in the ENABLE state when the slave is not ready and completes the write only after PREADY is asserted.

TC5 – Read with Wait States

Tests whether the master waits for PREADY during a read operation and correctly captures the requested data after the wait states.

TC6 – Error Handling (PSLVERR)

Tests the system's response to an invalid address and checks whether the slave correctly asserts PSLVERR.

TC7 – Burst / Back-to-Back Transfers

Tests multiple consecutive write and read transactions to verify that transfers can be performed back-to-back without incorrect APB control signals.

TC8 – Out-of-Range Address

Tests the behavior of the system when an address outside the intended valid range is accessed and checks for an error response.

TC9 – Reset Behavior

Tests whether asserting the active-low reset returns the master and APB control signals to their default states, including IDLE, PSEL = 0, and PENABLE = 0.

TC10 – Randomized Transactions

Performs 20 randomly generated read and write transactions with different addresses and data to stress-test the APB system and check its stability.

TASK 2:

Master:

module master(

input clk,

input rst,

input start,

input ready,

input [1:0] addr_in,

input rw_in,

input [7:0] data_in,

output reg valid,

output reg [1:0] addr,

output reg rw,

output reg [7:0] datain

);

parameter IDLE = 2'b00,

SEND = 2'b01,

WAIT = 2'b10;

reg [1:0] state,next_state;

always @(posedge clk or posedge rst)

begin

if(rst)

state <= IDLE;

else

state <= next_state;

end

always @(*)

begin

case(state)

IDLE:

if(start)

next_state = SEND;

else

next_state = IDLE;

SEND:

next_state = WAIT;

WAIT:

if(ready)

next_state = IDLE;

else

next_state = WAIT;

default:

next_state = IDLE;

endcase

end

always @(*)

begin

valid = 0;

case(state)

IDLE:

begin

valid = 0;

end

SEND:

begin

valid = 1;

addr = addr_in;

rw = rw_in;

datain = data_in;

end

WAIT:

begin

valid = 1;

addr = addr_in;

rw = rw_in;

datain = data_in;

end

endcase

end

endmodule

Slave:

module slave(

input clk,

input rst,

input valid,

input rw,

input [1:0] addr,

input [7:0] datain,

output reg ready,

output reg [7:0] dataout

);

parameter IDLE   = 2'b00,

ACCESS = 2'b01,

DONE   = 2'b10;

reg [1:0] state,next_state;

reg [7:0] mem [3:0];

always @(posedge clk or posedge rst)

begin

if(rst)

state <= IDLE;

else

state <= next_state;

end

always @(*)

begin

case(state)

IDLE:

if(valid)

next_state = ACCESS;

else

next_state = IDLE;

ACCESS:

next_state = DONE;

DONE:

if(valid)

next_state = DONE;

else

next_state = IDLE;

default:

next_state = IDLE;

endcase

end

always @(posedge clk)

begin

if(state==ACCESS)

begin

if(rw)

mem[addr] <= datain;

else

dataout <= mem[addr];

end

end

always @(*)

begin

ready = 0;

case(state)

IDLE:

ready = 0;

ACCESS:

ready = 0;

DONE:

ready = 1;

endcase

end

endmodule

Top:

module top(

input clk,

input rst,

input start,

input [1:0] addr,

input rw,

input [7:0] datain,

output [7:0] dataout,

output valid,

output ready

);

wire valid_wire;

wire ready_wire;

wire [1:0] addr_wire;

wire rw_wire;

wire [7:0] data_wire;

master M1(

.clk(clk),

.rst(rst),

.start(start),

.ready(ready_wire),

.addr_in(addr),

.rw_in(rw),

.data_in(datain),

.valid(valid_wire),

.addr(addr_wire),

.rw(rw_wire),

.datain(data_wire)

);

slave S1(

.clk(clk),

.rst(rst),

.valid(valid_wire),

.rw(rw_wire),

.addr(addr_wire),

.datain(data_wire),

.ready(ready_wire),

.dataout(dataout)

);

assign valid = valid_wire;

assign ready = ready_wire;

Endmodule

TASK 3: Handshaking protocol

Slave

FSM of Slave

FSM of Master

Master Slave handshaking protocol

Master FSM

| State | Name | valid | What happens? |
| --- | --- | --- | --- |
| 00 | IDLE | 0 | Wait for a start command. Nothing is sent. |
| 01 | SEND | 1 | Put addr, R/W, and datain on the bus. Start the transaction. |
| 10 | WAIT | 1 | Keep the signals stable and wait until the slave asserts ready. |

Slave FSM

| State | Name | ready | What happens? |
| --- | --- | --- | --- |
| 00 | IDLE | 0 | Wait for the master to assert valid. |
| 01 | ACCESS | 0 | Perform the requested read or write operation. |
| 10 | DONE | 1 | Assert ready to indicate the transaction is complete. |